Project Proposal: 4-Bit Processor Datapath (ALU, Registers, and Control FSM)
Group 7: Vansh , Varun, Shaunabh, Harsh

1. Motivation
Our labs covered binary arithmetic, combinational logic, FSMs, and SystemVerilog testbenching, but only separately. A CPU datapath ties all of them together and connects directly to the upcoming assembly and computer organization material and to SoCET’s RISCVBusiness core. Building a small ALU with registers and a control FSM shows how instructions turn into hardware actions. The design can be built in stages so we have a working milestone early and room for extensions.

2. High-Level Description
We will design a 4-bit processor datapath in SystemVerilog: an ALU (add, subtract, AND, OR, XOR, NOT, shifts) with zero, carry, and negative flags; a four-entry register file of 4-bit registers; and a control FSM that fetches and executes instructions from a small instruction ROM. We will define a compact 10-bit instruction format, hand-assemble a short program such as a counter or Fibonacci sequence, and run it. Each block will be verified with a testbench, and full programs will be simulated, then synthesized with Yosys and FreePDK45 to report area and timing. If an FPGA is available, we will demo the machine with register values on LEDs.

3. Division of Labor

| Member | Responsibility |
| --- | --- |
| Harsh | ALU (adder/subtractor, logic, shifter) and flag logic |
| Varun | Register file and instruction ROM |
| Vansh | Control FSM and instruction decoding |
| Shaunabh | Testbenches, program assembly, synthesis, and FPGA demo |

Everyone contributes to documentation, check-ins, and the final report, and each person reviews one teammate’s module.

Timeline

| Date | Milestone |
| --- | --- |
| 10/17 | Final proposal; block diagram, instruction format, and opcode table |
| 10/31 (Check-In 1) | ALU and register file written and passing testbenches |
| 11/14 (Check-In 2) | Control FSM integrated; first instructions executing in simulation |
| 11/29 (Check-In 3) | Full programs running; synthesis results collected; FPGA demo if available |
| 12/10 | Final presentation |
| 12/18 | Final report |

Risk plan: the ALU is the first milestone, so if the control FSM runs behind, we still deliver a verified, synthesized ALU with registers.

4. Final Deliverables

- SystemVerilog source for the ALU, register file, ROM, control FSM, and top level

- Testbenches for each block and a full-program simulation with labeled waveforms

- Instruction set table, block diagram, and FSM state diagram

- Hand-assembled demo programs with expected results

- Yosys/FreePDK45 synthesis report (area and timing)

- FPGA demo or a recorded simulation run

- Presentation slides and final report with usage instructions

MINI CPU Spec

Instruction formats (10 bits):

| TYPE | [9:6] | [5:4] | [3:2] | [1:0] |
| --- | --- | --- | --- | --- |
| R-type | opcode | rd | Rs | unused |
| I-type | opcode | rd | imm4 = [3:0] |  |
| Jump | opcode | unused | target = [3:0] |  |

Instruction Set:

| Opcode | Mnemonic | Operation | Flags updated |
| --- | --- | --- | --- |
| 0000 | NOP | No operation | none |
| 0001 | LDI rd, imm4 | rd ← imm4 | none |
| 0010 | MOV rd, rs | rd ← rs | none |
| 0011 | ADD rd, rs | rd ← rd + rs | Z, C, N |
| 0100 | SUB rd, rs | rd ← rd − rs (computed as rd + ~rs + 1) | Z, C, N |
| 0101 | AND rd, rs | rd ← rd & rs | Z, N |
| 0110 | OR rd, rs | rd ← rd ¦ rs | Z, N |
| 0111 | XOR rd, rs | rd ← rd ^ rs | Z, N |
| 1000 | NOT rd, rs | rd ← ~rs | Z, N |
| 1001 | SHL rd | rd ← rd << 1 (C = old bit 3) | Z, C, N |
| 1010 | SHR rd | rd ← rd >> 1, logical (C = old bit 0) | Z, C, N |
| 1011 | CMP rd, rs | Computes rd − rs, no writeback | Z, C, N |
| 1100 | JMP target | PC ← target | none |
| 1101 | BZ target | if Z = 1, PC ← target | none |
| 1110 | BNZ target | if Z = 0, PC ← target | none |
| 1111 | HLT | Stop in the HALT state until reset | none |

Architecture: 4-bit data path; four general registers R0–R3 (2-bit address); a 4-bit program counter (PC); a 16-entry × 10-bit instruction ROM initialized at synthesis; no data memory. On reset the PC, registers, and flags are 0 and the FSM is in FETCH.

Flags: Z = result is zero. N = bit 3 of the result. C = carry out of the 4-bit adder; for SUB and CMP, C = 1 means no borrow (rd ≥ rs, unsigned). SHL/SHR place the bit shifted out in C.

Control FSM (4 states):

- FETCH: IR ← ROM[PC]; PC ← PC + 1 (wraps mod 16).

- DECODE: Generate control signals (register write enable, ALU operation, flag update, branch select) from the opcode.

- EXECUTE: ALU operation, register write, flag update, or branch (loads the PC).

- HALT: Entered by HLT; no further activity until reset.

Every instruction takes three cycles (FETCH, DECODE, EXECUTE), except HLT, which stays in HALT.

Demo program (3 × 4 by repeated addition; final R0 = 12 = 0xC):

| Addr | Assembly | Binary | Hex |
| --- | --- | --- | --- |
| 0 | LDI R0, 0 | 0001 00 0000 | 040 |
| 1 | LDI R1, 3 | 0001 01 0011 | 053 |
| 2 | LDI R2, 4 | 0001 10 0100 | 064 |
| 3 | LDI R3, 1 | 0001 11 0001 | 071 |
| 4 | ADD R0, R1 | 0011 00 01 00 | 0C4 |
| 5 | SUB R2, R3 | 0100 10 11 00 | 12C |
| 6 | BNZ 4 | 1110 00 0100 | 384 |
| 7 | HLT | 1111 00 0000 | 3C0 |

FPGA demo plan (ECE 270 simulator): a debounced push button single-steps the clock and a second button resets. Switches select which register is shown; LEDs/7-segment display the selected register, with the PC and FSM state on spare outputs. Verification: run the demo program plus a directed test for every opcode, including flag behavior and both BZ outcomes (taken and not taken).